Day 1 — Reverse Engineering Analysis: Lapcare 2.4G Wireless Mouse (Model: WL 300)

1. Physical Inspection — Studying the Product

Start with the device without opening it.

Documented Physical Specifications (Lapcare WL 300):

Product Name / Model: Lapcare 2.4G Wireless Mouse / Model: WL 300

Manufacturer: Lapcare (Made in China)

Approximate Age / Version: PCB Date Marking: 2016/12/05

External Dimensions: Compact ergonomic palm-grip (~100mm x 60mm x 38mm)

Buttons / Switches: Left click, Right click, Scroll wheel click (middle button), DPI switch

LEDs / Displays: Optical LED light source for surface tracking

Connectors: 2-pin white battery wire connector (CON1) inside housing

Power Source: Single AAA/AA Battery Compartment (Rating: 5V === 100mA)

Ports & Sensors: Underside optical tracking lens window & ON/OFF slide switch

Mechanical Interfaces: Plastic snap-fits, screw mounts, scroll wheel encoder bracket

Labels and Markings: Lapcare logo, Model WL 300, CE / FCC certifications, Serial number

Visible Screws / Seams: 1 primary housing retention screw beneath bottom shell

Functional Analysis Questions:

A. What is this product supposed to do?

Provide wireless cursor movement, clicking capability, and scrolling navigation for personal computing.

B. What does the user expect?

For a 2.4G Wireless Mouse:

Cable-free operation → low latency tracking → long battery life → ergonomic comfort → durable tactile feedback.

C. What engineering decisions can we already infer?

Inferred Architecture:

Battery Input → Power Slide Switch → Filtering Capacitors → Optical Sensor & Glob-Top Controller IC → 2.4GHz RF Sub-board Antenna

Formulating a functional hypothesis prior to teardown establishes the core objective of reverse engineering.

2. Product Literature & Specification Review

Investigation of component evolution and hardware iterations:

| Parameter | Standard Wired Mouse | Lapcare WL 300 Wireless Mouse |
| --- | --- | --- |
| Processor / Controller | Wired USB Optical IC | Glob-top Integrated MCU + RF IC |
| Power Source | 5V via USB Cable | 1.5V/3V Battery via internal CON1 connector |
| Connectivity | Direct USB Cable | 2.4GHz Wireless RF Sub-board module |
| PCB Technology | Single-sided FR2/Paper Phenolic | Single-sided Phenolic PCB (QKTR-WK-611-4D) |
| Power Management | Continuous host power | Physical ON/OFF Slide Switch & Sleep Mode |
| Sensors | Standard Red Optical Sensor | Integrated Optical Navigation Sensor |
| Mechanical Switches | 3 Standard Micro-switches | 4 Micro-switches + Mechanical Rotary Encoder |

Engineering Reasoning for Design Changes:

Elimination of cables for desktop mobility

Direct chip-on-board (Glob-top IC) to minimize assembly costs

Modular RF sub-card soldering to reuse PCB layout across model variants

Power management integration to extend battery life

3. Product Analysis — Group Discussion

Purpose: Provides low-cost, cord-free desktop cursor control.

User expectations: Smooth tracking, immediate click response, portable size, affordable price.

Engineering Trade-offs: Use of paper-phenolic PCB material and glob-top IC encapsulation reduces repairability but drastically cuts unit cost.

4. Guided Teardown Preparation & ESD Precautions

Teardown Sequence: Document → Photograph → Open Shell → Disconnect Power → Record Components.

Teardown Tools Utilized

Precision Screwdriver

Plastic Pry Tool / Spudger

Tweezers

Multimeter

Magnifying Glass

ESD Rule: Always discharge static energy before touching the mouse PCB to prevent damage to the optical IC and RF controller.

Day 2 — Hardware Disassembly & Component Inventory

5. Internal Hardware Inventory (BOM) — Lapcare WL 300

| Ref./Location | Component | Package | Marking | Function | Notes |
| --- | --- | --- | --- | --- | --- |
| U1 | MCU / Optical RF IC | Glob-top COB | Black Epoxy Encapsulation | Main controller & optical processor | Mounted on top side of PCB |
| S1 | Power Slide Switch | Through-Hole SPDT | ON / OFF | Disconnects battery supply rail | Located near battery compartment |
| SW1, SW2 | Micro-switches | 3-pin Snap Action | Red Plunger / Black Body | Left and Right mouse click inputs | Tactile feedback switches |
| SW3 | Rotary Mechanical Encoder | Through-Hole Metal Frame | 8-11 / Metal Housing | Scroll wheel rotation detection | Houses scroll wheel shaft |
| RF Sub-card | 2.4G RF Transmitter | Vertical Daughterboard | Red PCB / PCB Trace Antenna | Wireless 2.4GHz signal transmission | Soldered perpendicular to main PCB |
| XTAL1 | Crystal Oscillator | Cylindrical DIP | Metal Can | Clock timing for RF / MCU | Mounted on RF daughterboard |
| C1, C2 | Electrolytic Capacitors | Radial Leaded | 100uF 10V | Power supply filtering | Smooths DC supply from battery |
| LED1 | Optical Illumination LED | 5mm DIP Clear | Standard Red Lens | Provides surface reflection for tracking | Positioned next to optical lens |
| CON1 | Battery Header | 2-pin JST White Header | CON1 / Red-Black Wire | Interface for battery compartment | Main power entry point |

Day 3 — Component Functional Analysis & Architecture

6. System Functional Analysis

| Component | Electrical Role | Functional Role | Connected To |
| --- | --- | --- | --- |
| Glob-top IC (U1) | Digital Signal Processing | Processes surface motion and button clicks | Switches, Optical Sensor, RF Sub-board |
| RF Sub-card | High Frequency Modulation | Transmits 2.4GHz wireless data packets | Glob-top IC, Trace Antenna, Crystal |
| Rotary Encoder (SW3) | Quadrature Pulse Generation | Detects scroll wheel movement direction | Glob-top IC |
| Micro-switches | Binary Contact Closure | Sends left/right/middle click triggers | Glob-top IC |
| Power Switch & Capacitors | DC Regulation & Filtering | Controls system power & eliminates ripple | Battery → CON1 → System Rail |

System Architecture Block Diagram

┌──────────────────────────────┐
             │   BATTERY POWER COMPARTMENT  │
             └──────────────┬───────────────┘
                            │
             ┌──────────────▼───────────────┐
             │  ON/OFF Switch & Capacitors │
             └──────────────┬───────────────┘
                            │
       ┌────────────────────┼────────────────────┐
       │                    │                    │
       ▼                    ▼                    ▼
┌──────────────┐     ┌──────────────┐     ┌──────────────┐
│ Optical LED  │     │ Microswitches│     │ Wheel Rotary │
│ & Tracking   │     │ (Left/Right) │     │ Encoder      │
└──────┬───────┘     └──────┬───────┘     └──────┬───────┘
       │                    │                    │
       └────────────────────┼────────────────────┘
                            │
             ┌──────────────▼───────────────┐
             │  Glob-Top MCU / Optical IC   │
             └──────────────┬───────────────┘
                            │
             ┌──────────────▼───────────────┐
             │ 2.4GHz RF Module & Antenna   │
             └──────────────┬───────────────┘
                            │ (Wireless Link)
                            ▼
                  [ Host Computer USB ]

Five Levels of Engineering Analysis

Level 1 — Observation: "I see a single-sided brown PCB, micro-switches, scroll wheel, black glob-top IC, and a battery connector."

Level 2 — Identification: "This board is model QKTR-WK-611-4D manufactured in Dec 2016 for Lapcare."

Level 3 — Specification: "Operates on 5V/100mA rating with a 2.4GHz RF sub-board and 100uF 10V filtering capacitors."

Level 4 — Functional Understanding: "The optical LED illuminates the surface while the glob-top IC processes motion and transmits wireless signals via the RF sub-board."

Level 5 — Engineering Reasoning: "The designer selected a glob-top IC and vertical RF sub-board module to drastically reduce PCB layer count and assembly costs for high-volume manufacturing."

Student Deliverables & Conclusions

Lapcare WL 300 External Inspection Sheet

Wired vs. Wireless Component Comparison

Teardown Photo Log & Mechanical Assembly Map

Complete Wireless Mouse Bill of Materials (BOM)

2.4GHz Wireless Mouse Functional Block Diagram

Final Reverse-Engineering Report & Design Conclusions